### Dispatch Instructions for Sending Frozen Muscle Biopsies to Cellular Pathology, RVI

Dispatch Instructions – Please read carefully

Muscle biopsy samples and accompanying forms should be sent to:

Specimen Reception

Department of Cellular Pathology 			Tel: 0191 2829133

Level 3 New Victoria Wing

Royal Victoria Infirmary

Queen Victoria Road

Newcastle

NE1 4LP

# Muscle Biopsy - Dispatch Instructions

Muscle biopsies are extremely precious specimens and it is important that these instructions are followed precisely.  Failure to follow the instructions may result in the specimen becoming lost, delayed, damaged or arriving at a time when the laboratory is closed.

IF YOU ARE IN ANY DOUBT PLEASE CONTACT THE LABORATORY FOR ADVICE

TELEPHONE: 0191 2829133

MON – FRI 08.00 – 18.00

Contact the Lab Prior to Sending Muscle Biopsies

Specimen Requirements – Muscle Biopsies

Any paperwork and/or non-frozen specimens should be included in/with the package.

All frozen specimens should be placed together in a sealed plastic bag and on top of the dry ice.

Please note that we are not able to accept muscle biopsies from patients known or suspected of being infected with a Hazard Group 3 or 4 organism, such as TB, HIV or Hepatitis B and C.

Packaging Requirements for Muscle Biopsies– Dry Ice Is Essential

The muscle sample must be sent on dry ice, using a courier for ‘door to door’ delivery.

(eg TNT or PDP Pharmaceutical couriers).

Pack the muscle samples in a thick-walled (e.g. 2") polystyrene container, enclosed in a cardboard box.  Suitable containers (e.g. the Biotherm range) which meet all current transport regulations can be obtained from

http://www.intelsius.com/products/temperature-controlled-packaging-and-cold-chain-solutions/biotherm  or http://www.cool-logistics.com/products.aspx

Fill with sufficient dry ice to last 48 hours (e.g. several kg).  The optimal amount of dry ice depends on several factors, including the internal volume of the insulated container, the wall thickness and so on.  Examples can be found at: www.cool-logistics.com.au/dryice/CL_DryIceCalculator.xls

Do not try to pack the dry ice with polystyrene chips, bubble wrap or any other material that will mix with the dry ice during transit and insulate the sample from the dry ice.

Documents to Accompany Specimens

When you send the samples, please include:

- Referral form

- A covering letter, with the names and addresses of any additional people who will require copies of letters or reports e.g. GPs

- A recent clinical summary letter

Laboratory Address for Sending Muscle Biopsies

Address the package to:

Specimen Reception

Department of Cellular Pathology 				Tel: 0191 2829133

Level 3 New Victoria Wing

Royal Victoria Infirmary

Queen Victoria Road

Newcastle

NE1 4LP

Please put address labels on more than one side because the courier will usually apply a company label to the top surface and it may obscure delivery address

Do not send samples on dry ice to arrive on a Friday.  Delays may well move the delivery of your package into the weekend and your sample may thaw and be lost.